Manufacturer:

GDUK Door Solutions,

Securiparc House,

Wimsey Way,

Alfreton,

Derbyshire,

DE55 4LS.

Tel: 0800 316 6994.

Estimating: estimating@gdukds.com

Web: www.gdukds.com

Automatic Operator:

Gilgen SL-SRM Curved Sliding Door Operator with 24V DC motor with microprocessor controller which allows extensive options for opening width, electric locking, summer/ winter modes, exit only, manual control, speed of opening and the length of time the door is open.

Operator height 200mm, depth 250mm. Maximum door leaf weight of 120kg.

Activation and control system per doorset:

Key switch with 6 positions. Configured for off, automatic, night, exit only and hold open operating modes.

- Exit:   1no.Combined & Monitored activation and safety sensor EN 16005:2012

- Entry: 1no.Combined & Monitored activation and safety sensor EN 16005:2012

Safety devices per door:

- 1no. Battery Pack for fail open or closed in the event of power failure.

- Option that enables up to 30 minutes operation (subject to use) in the event of

a mains failure. This fail safe system is automatically tested at least once every 24 hours.

- Monitored presence sensor back edge safety.

Locking:

- Electromechanical “Carlock” running-carriage locking device reliably prevents the door leaves from sliding open. The built-in gear motor guarantees a high locking/unlocking force, tested to over 500,000 operating cycles. (delete as appropriate)

- Manual lock (delete as appropriate)

Configuration

360 Degrees

180 Degrees

Convex

Concave

Materials:

Doors:

- Formed from Gilgen PSXP aluminium profiles with vertical interlocking and sealing gaskets. Slim 40mm vertical profiles with 70mm bottom rail.

- Doors  main closing edge will be based on the “tongue and groove principle.

- Glazing (delete as appropriate)

  - 6.8mm single glazed laminated

  - 10mm single glassed toughened

Side screens

- Formed from Gilgen PSXP aluminium profiles with vertical interlocking and sealing gaskets. Slim 40mm vertical profiles with 70mm bottom rail.

- Glazing (delete as appropriate)

  - 6.8mm single glazed laminated

  - 10mm single glassed toughened

- Radius:                                                 mm

- Clear Opening Width:		mm

- Clear Opening Height:		mm

Finishes:

- All visible operator surfaces to be Powder Coated to a standard RAL colour. (delete as appropriate)

- Standard anodized finish. (delete as appropriate)

Signs:

- Standard signage to EN16005.

- Manifestation not included.

Associated works Per Door Set:

- 1 no. 240v 13A LIVE neon lit fused spur supply, protected by a 30mA RCD must be provided prior to installation. In compliance with BS EN16005 the mains connection must be capable of disconnection to safeguard against unintentional and unauthorised reactivation. Positioned above operator by others.

- Connection to the fire alarm system (if required) would require one pair of normally open volt free contacts to be supplied terminating at drive unit location by others.

- Any concealed containment (inc. draw wires) must be undertaken by others. Cabling to activation devices will otherwise be fitted with surface mini trunking at a minimum distance to suit industry standards.

  - Notes:   Installation carried out by the manufacturer’s engineers.  Installed to and complying with EN16005